Marstek Device Open API（Rev 1.0）

The Local API is provided “as is” for local use only. Use at your own risk. Marstek is not liable for any damages, data loss, or legal issues caused by your use of the API. You are responsible for lawful and appropriate use.

# Ⅰ. Preface

Welcome!

This document provides an introduction to the Open API for Marstek devices, which is available to device owners and enables integration with third-party systems.

While Marstek offers an official mobile app and cloud services, this Open API is designed for advanced users who wish to gain greater control over their devices and seamlessly integrate them into other management platforms.

# Ⅱ. General Description

Marstek devices communicate with third-party systems over a Local Area Network (LAN). Before using this API, please ensure that:

The Marstek device is properly connected to your home network.

The Open API feature has been enabled via the Marstek mobile app.

Please note that different Marstek models may support only a subset of the commands described in this documentation. Additionally, enabling the Open API may cause certain built-in features of the device to be disabled in order to prevent command conflicts. For detailed information about supported commands and any functional limitations for specific models, please refer to Chapter 4 of this document.

## 2.1 Protocol Format

The protocol utilizes the JSON format, with commands primarily categorized into query commands, configuration commands, and certain special commands.

Command Format

| Property | Type | Description |
| --- | --- | --- |
| id | number or string | An identifier established by the Client. |
| method | string | A Structured value that holds the parameter values to be used during the invocation of the method. |
| params | object | Parameters that the method takes. |

Example

{

"id"

:

0

,

"method"

:

"string"

,

"params"

:

{

"id"

:

0

}

}

Device Response Format

| Property | Type | Description |
| --- | --- | --- |
| id | number or string | An identifier established by the Client. |
| src | string | Name of the source of the request. |
| result | object | This member is required on success. |
| error | object | This member is required on error. |

Error code

| Code | Message | Meaning |
| --- | --- | --- |
| -32700 | Parse error | Invalid JSON was received by the server. An error occurred on the server while parsing the JSON text. |
| -32600 | Invalid Request | The JSON sent is not a valid Request object. |
| -32601 | Method not found | The method does not exist / is not available. |
| -32602 | Invalid params | Invalid method parameter(s). |
| -32603 | Internal error | Internal JSON-RPC error. |
| -32000 to -32099 | Server error | Reserved for implementation-defined server-errors. |

The remainder of the space is available for application defined errors.

Example response on success

{

"id"

:

0

,

"src"

:

"device"

,

"result"

:

{

"id"

:

0

}

}

Example response on error

{

"id"

:

0

,

"src"

:

"device"

,

"error"

:

{

"code"

:

-

32700

,

"message"

:

"Parse error"

}

}

## 2.2 API over UDP

2.2.1 First-Time Use

When users first use the Open API service, they need to follow the configuration process below:

Connect the device to power and turn it on;

Use the Marstek APP to connect and bind the device, and configure the WiFi network for the device or connect it to the Ethernet;

Enable the device's API feature in the APP, and set the UDP port number. The default port number is 30000, and the recommended port number is between 49152 and 65535.

After completing the above operations, the Marstek device can normally receive UDP commands from the same local area network.

2.2.2 Discovering Devices

To discover Marstek devices within the LAN, a UDP broadcast is utilized. The broadcast content is as follows:

{

"id"

:

0

,

"method"

:

"Marstek.GetDevice"

,

"params"

:

{

"ble_mac"

:

"0"

}

}

If there are Marstek devices within the LAN, taking Venus C as an example, the following response will be received

{

"id"

:

0

,

"src"

:

"VenusC-123456789012"

,

"result"

:

{

"device"

:

"VenusC"

,

"ver"

:

111

,

"ble_mac"

:

"123456789012"

,

"wifi_mac"

:

"123456789012"

,

"wifi_name"

:

"MY_HOME"

,

"ip"

:

"192.168.1.11"

}

}

The device's IP address can be directly obtained from the Marstek APP or the home router. If this functionality is to be used on a long-term basis, it is recommended to configure the device with a static IP address.

# Ⅲ. Components

This chapter mainly introduces the components and services supported by the Marstek device.

## 3.1 Marstek

Marstek contains some basic information about the product, and is mainly used for discovering devices and querying basic device information.

Marstek.GetDevice: Locate Marstek devices on the local area network.

3.1.1 Marstek.GetDevice

Sending:

| Property(params) | Type | Description |
| --- | --- | --- |
| ble_mac | string | Valid mac，can be used to identify a specific device. |

Response:

| Property(result) | Type | Description |
| --- | --- | --- |
| device | string | Device model |
| ver | number | Device firmware version |
| ble_mac | string | Bluetooth MAC |
| wifi_mac | string | WiFi MAC |
| wifi_name | string | WiFi name |
| ip | string | Device IP |

Example:

Sending:

{

"id"

:

0

,

"method"

:

"Marstek.GetDevice"

,

"params"

:

{

"ble_mac"

:

"123456789012"

}

}

Response:

{

"id"

:

0

,

"src"

:

"VenusC-123456789012"

,

"result"

:

{

"device"

:

"VenusC"

,

"ver"

:

111

,

"ble_mac"

:

"123456789012"

,

"wifi_mac"

:

"012123456789"

,

"wifi_name"

：

"MY_HOME"

,

"ip"

:

"192.168.1.11"

}

}

## 3.2 WiFi

The WiFi component is mainly used for configuring the device's WiFi and obtaining the device's basic network information.

Wifi.GetStatus: Obtain the device's basic network information.

3.2.1 Wifi.GetStatus

Sending:

| Property (params) | Type | Description |
| --- | --- | --- |
| id | number | ID of Instance |

Response:

| Property (result) | Type | Description |
| --- | --- | --- |
| id | number | ID of Instance |
| wifi_mac | string | WiFi MAC |
| ssid | string or null | WiFi name |
| rssi | number | WiFi signal strength |
| sta_ip | string or null | Device IP |
| sta_gate | string or null | Gateway |
| sta_mask | string or null | Subnet mask |
| sta_dns | string or null | DNS |

Example:

Sending:

"Wifi.GetStatus",

0

Response:

{

"id"

:

1

,

"src"

:

"VenusC-mac"

,

"result"

:

{

"id"

:

0

,

"ssid"

:

"Hame"

,

"rssi"

:

-

59

,

"sta_ip"

:

"192.168.137.41"

,

"sta_gate"

:

"192.168.137.1"

,

"sta_mask"

:

"255.255.255.0"

,

"sta_dns"

:

"192.168.137.1"

}

}

## 3.3  Bluetooth

The BLE (Bluetooth) component can view the Bluetooth-related information of the device.

BLE.GetStatus: Check the Bluetooth connection status of the device.

3.3.1 BLE.GetStatus

Sending：

| Property (params) | Type | Description |
| --- | --- | --- |
| id | number | ID of Instance |

Response：

| Property (result) | Type | Description |
| --- | --- | --- |
| state | string | Bluetooth state |
| ble_mac | string | Bluetooth MAC |

Example：

Sending:

"BLE.GetStatus",

0

Response:

{

"id"

:

1

,

"src"

:

"VenusC-123456789012"

,

"result"

:

{

"id"

:

0

,

"state"

:

"connect"

,

"ble_mac"

:

"123456789012"

}

}

## 3.4  Battery

The Bat (Battery) component contains basic information about the device's battery.

Bat.GetStatus: Query the device's battery information and operating status.

3.4.1 Bat.GetStatus

Sending:

| Property (params) | Type | Description |
| --- | --- | --- |
| id | number | ID of Instance |

Response:

| Property (result) | Type | Description |
| --- | --- | --- |
| id | number | ID of Instance |
| soc | string | soc |
| charg_flag | boolean | Charging permission flag |
| dischrg_flag | boolean | Discharge permission flag |
| bat_temp | number or null | Battery temperature, [°C] |
| bat_capacity | number or null | Battery remaining capacity, [Wh] |
| rated_capacity | number or null | Battery rated capacity, [Wh] |

Example:

Sending:

"Bat.GetStatus",

0

Response:

{

"id"

:

1

,

"src"

:

"VenusC-mac"

,

"result"

:

{

"id"

:

0

,

"soc"

:

98

,

"charg_flag"

:

true

,

"dischrg_flag"

:

true

,

"bat_temp"

:

25.0

,

"bat_capacity"

:

2508.0

,

"rated_capacity"

:

2560.0

}

}

## 3.5  PV

The PV (Photovoltaic) component contains the photovoltaic information connected to the device.

PV.GetStatus: Query the device's connected photovoltaic information and power generation status.

3.5.1 PV.GetStatus

Sending:

| property (params) | Type | Description |
| --- | --- | --- |
| id | number | ID of Instance |

Response:

| Property (result) | Type | Description |
| --- | --- | --- |
| id | number | ID of Instance |
| pv_power | number | Photovoltaic charging power, [W] |
| pv_voltage | number | Photovoltaic charging voltage, [V] |
| pv_current | number | Photovoltaic charging current, [A] |

Example：

Sending:

{

"id"

:

1

,

"method"

:

"PV.GetStatus"

,

"params"

:

{

"id"

:

0

}

}

Response:

{

"id"

:

1

,

"src"

:

"VenusC-mac"

,

"result"

:

{

"id"

:

0

,

"pv_power"

:

580.0

,

"pv_voltage"

:

40.0

,

"pv_current"

:

12.0

}

}

## 3.6 ES

The ES (Energy System) component contains the device's basic power information and energy statistics, and can configure or monitor the device's operating status.

ES.GetStatus: Query the device's basic electrical energy information.

ES.SetMode: Configure the device's operating mode.

ES.GetMode: Get information about the operating mode of the device.

3.6.1 ES.GetStatus

Sending:

| Property (params) | Type | Description |
| --- | --- | --- |
| id | number or null | ID of Instance |

Response:

| Property (result) | Type | Description |
| --- | --- | --- |
| id | number or null | ID of Instance |
| bat_soc | number or null | Total battery SOC, [%] |
| bat_cap | number or null | Total battery capacity, [Wh] |
| Property (result) | Type | Description |
| pv_power | number or null | Solar charging power, [W] |
| ongrid_power | number or null | Grid-tied power, [W] |
| offgrid_power | number or null | Off-grid power, [W] |
| bat_power | number or null | Battery power, [W] |
| total_pv_energy | number or null | Total solar energy generated, [Wh] |
| total_grid_output_energy | number or null | Total grid output energy, [Wh] |
| total_grid_input_energy | number or null | Total grid input energy, [Wh] |
| total_load_energy | number or null | Total load (or off-grid) energy consumed, [Wh] |

Example：

Sending:

{

"id"

:

1

,

"method"

:

"ES.GetStatus"

,

"params"

:

{

"id"

:

0

}

}

Response:

{

"id"

:

1

,

"src"

:

"VenusC-mac"

,

"result"

:

{

"id"

:

0

,

"bat_soc"

:

98

,

"bat_cap"

:

2560

,

"pv_power"

:

0

,

"ongrid_power"

:

100

,

"offgrid_power"

:

0

,

"bat_power"

:

0

,

"total_pv_energy"

:

0

,

"total_grid_output_energy"

:

844

,

"total_grid_input_energy"

:

1607

,

"total_load_energy"

:

0

}

}

3.6.2 ES.SetMode

Sending:

| Property (params) | Type | Description |
| --- | --- | --- |
| id | number | ID of Instance |
| config | object | Config Parameters |

Object: config

| Property(config) | Type | Description |
| --- | --- | --- |
| mode | string | Device power generation mode, including the following modes： "Auto"；"AI"；"Manual"；"Passive". |
| auto_cfg | object | Configuration parameters for Auto mode |
| ai_cfg | object | Configuration parameters for AI mode |
| manual_cfg | object | Configuration parameters for Manual mode |
| passive_cfg | object | Configuration parameters for Passive mode |

Object: auto_cfg

| Property (auto_cfg) | Type | Description |
| --- | --- | --- |
| enable | number | ON: 1; OFF: Set another mode |

Object: ai_cfg

| Property (ai_cfg) | Type | Description |
| --- | --- | --- |
| enable | number | ON: 1; OFF: Set another mode |

Object: manual_cfg

| Property (manual_cfg) | Type | Description |
| --- | --- | --- |
| time_num | number | Time period serial number, Venus C/E supports 0-9 |
| start_time | string | Start time, hours: minutes, [hh:mm] |
| end_time | string | End time, hours: minutes, [hh:mm] |
| Property (manual_cfg) | Type | Description |
| week_set | number | Week, a byte 8 bits, the low 7 bits effective, the highest invalid, 0000 0001 (1) on behalf of Monday open, 0000 0011 (3) on behalf of Monday and Tuesday open, 0111 1111 (127) on behalf of a week |
| power | number | Setting power,[W] |
| enable | number | ON: 1; OFF: 0 |

Object: passive_cfg

| Property(passive_cfg) | Type | Description |
| --- | --- | --- |
| power | number | Setting power,[W] |
| cd_time | number | Power countdown,[s] |

Response：

| Property（result） | Type | Description |
| --- | --- | --- |
| id | number | ID of Instance |
| set_result | boolean | "true":succeeded in setting; "false":failed in setting |

Example：

Sending:

/* Auto Mode Example */

{

"id"

:

1

,

"method"

:

"ES.SetMode"

,

"params"

:

{

"id"

:

0

,

"config"

:

{

"mode"

:

"Auto"

,

"auto_cfg"

:

{

"enable"

:

1

}

}

}

}

/* AI Mode Example */

{

"id"

:

1

,

"method"

:

"ES.SetMode"

,

"params"

:

{

"id"

:

0

,

"config"

:

{

"mode"

:

"AI"

,

"ai_cfg"

:

{

"enable"

:

1

}

}

}

}

/* Manual Mode Example */

{

"id"

:

1

,

"method"

:

"ES.SetMode"

,

"params"

:

{

"id"

:

0

,

"config"

:

{

"mode"

:

"Manual"

,

"manual_cfg"

:

{

"time_num"

:

1

,

"start_time"

:

"08:30"

,

"end_time"

:

"20:30"

,

"week_set"

:

127

,

"power"

:

100

,

"enable"

:

1

}

}

}

}

/* Passive Pattern Example */

{

"id"

:

1

,

"method"

:

"ES.SetMode"

,

"params"

:

{

"id"

:

0

,

"config"

:

{

"mode"

:

"Passive"

,

"passive_cfg"

:

{

"power"

:

100

,

"cd_time"

:

300

}

}

}

}

Response:

{

"id"

:

1

,

"src"

:

"Venus-mac"

,

"result"

:

{

"id"

:

0

,

"set_result"

:

ture

}

}

3.6.3 ES.GetMode

Sending:

| Property (params) | Type | Description |
| --- | --- | --- |
| id | number or null | ID of Instance |

Response:

| Property (result) | Type | Description |
| --- | --- | --- |
| id | number or null | ID of Instance |
| mode | number or null | Auto：Auto mode; AI: AI mode; Manual: manual mode; Passive: Passive control mode |
| ongrid_power | number or null | Grid-tied power, [W] |
| offgrid_power | number or null | Off-grid power, [W] |
| bat_soc | number or null | SOC,[%] |

Example：

Sending:

{

"id"

:

0

,

"method"

:

"ES.GetMode"

,

"params"

:

{

"id"

:

0

}

}

Response:

{

"id"

:

0

,

"src"

:

"VenusC-mac"

,

"result"

:

{

"id"

:

0

,

"mode"

:

"Passive"

,

"ongrid_power"

:

100

,

"offgrid_power"

:

0

,

"bat_soc"

:

98

}

}

## 3.7 EM

The Energy Meter (EM) module contains status information and power measurement data from the energy meter, or data obtained from the current transformer (CT).

EM.GetStatus: Queries the basic status and data information of the energy meter.

3.7.1 EM.GetStatus

Sending:

| Property (params) | Type | Description |
| --- | --- | --- |
| id | number or null | ID of Instance |

Response：

| Property (result) | Type | Description |
| --- | --- | --- |
| id | number or null | ID of Instance |
| ct_state | number or null | CT (Current Transformer) status: 0: Not connected 1: Connected |
| a_power | number or null | Phase A power,[W] |
| b_power | number or null | Phase B power,[W] |
| c_power | number or null | Phase C power,[W] |
| total_power | number or null | Total power,[W] |

Sending:

{

"id"

:

1

,

"method"

:

"EM.GetStatus"

,

"params"

:

{

"id"

:

0

}

}

Response:

{

"id"

:

1

,

"src"

:

"VenusC-mac"

,

"result"

:

{

"id"

:

0

,

"ct_state"

:

0

,

"a_power"

:

0

,

"b_power"

:

0

,

"c_power"

:

0

,

"total_power"

:

0

}

}

# . Devices

This chapter will describe the extent of support for the components and services in this API documentation by different Marstek devices, as well as some proprietary information.

4.1 Venus C/E

Marstek

WiFi

Bluetooth

Battery

ES

EM

4.2 Venus D

Marstek

WiFi

Bluetooth

Battery

PV

ES

EM

# . Change Logs

This chapter explains the change log for the API documentation.

● 2025-08-09Modified: Version number Rev 1.0